The classification looks solid — brown ground, green vegetation, white/yellow bird contacts. The "fat" ground band (especially visible in images 1 and 4) is caused by the class threshold being too loose — it's catching low noise, grass tufts, and points sitting just above the true surface.

Settings to tighten the ground class

The key parameter is CSF Class threshold — it's the band (in metres) around the cloth surface within which points get labelled ground. In your build it also directly drives the final ground mask (|nZ| ≤ class_threshold).

| Parameter | Current default | Change to | Effect |
| --- | --- | --- | --- |
| Class threshold | 0.10 m | 0.05 m | ← main fix. Halves the ground band thickness |
| Cloth resolution | 0.50 m | 0.30 m | Cloth follows terrain tighter, won't bridge over low vegetation |
| Iterations | 500 | 800 | Cloth settles properly into the true surface |
| Rigidness | 2 | 1 | Keep at 1 — your scenes have real slopes; stiffer cloth would lift off and catch more |
| Pre-thin voxel | 0.15 m | 0.10 m | Keeps finer ground detail so the surface is accurate |

Recommended starting combo

Method:            CSF

Cloth resolution:  0.30 m

Class threshold:   0.05 m

Rigidness:         1

Iterations:        800

Pre-thin voxel:    0.10 m

Slope smooth:      ✓ enabled

Reality check

A terrestrial scan of natural ground (soil, grass, gravel) has genuine 3–8 cm roughness — you can't get a perfectly thin 1-point band, and you shouldn't try. If you push class threshold below ~0.03 m you'll start losing real ground points.

If specific spots still over-catch after this:

- Slopes catching vegetation (image 2) → drop cloth resolution to 0.25 m

- Dense low scrub merging into ground (image 3) → that's genuinely ambiguous; CSF can't separate 5 cm scrub from ground reliably — would need a post-pass on intensity or return count